Lab 2 – Week 2

AHB Protocol

Design Code

Master

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Design Name:

// Module Name: ahb_master

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

//////////////////////////////////////////////////////////////////////////////////

/////////////// This is the code for the MASTER //////////////////////////////////

////////////// The working and code is written as per the state model ///////////

`define OKAY  1'b0

`define ERROR 1'b1

module ahb_master(

input clk,

input hresetn,

input enable,

input hresp,

input hready,

input [31:0] hrdata,

input [31:0] in_hwdata,

input [31:0] in_haddr,

input [2:0] in_hsize,

input [2:0] in_hburst,

input [1:0] in_hsel,

input in_hwrite,

input [1:0] in_htrans,

output logic [31:0] hwdata,

output logic [31:0] haddr,

output logic [2:0] hsize,

output logic [2:0] hburst,

output logic [1:0]hsel,

output logic hwrite,

output logic [1:0] htrans,

output logic [31:0] out_hrdata

//output logic hreadyout

);

typedef enum  {idle = 0, busy = 1, error=2} state_type;

state_type master_state, next_state;

///////////////////////////////////////////////////////////////////////////////////

always_ff@(posedge clk)

begin

if(!hresetn)

master_state <= idle;

else

master_state <= next_state;

end

always_comb

begin

case(master_state)

idle:

begin

if(enable==1)

begin

next_state= busy;

hwdata= in_hwdata;

haddr= in_haddr;

hsize= in_hsize;

hburst= in_hburst;

hsel= in_hsel;

hwrite= in_hwrite;

htrans= in_htrans;

end

else next_state= idle;

//        if(hresp==`ERROR)

//            next_state= error;

end

busy:

begin

if(hresp==`ERROR)

next_state= error;

else next_state=busy;

if (hready==1)

begin

out_hrdata= hrdata;

next_state= idle;

end

else next_state=busy;

if(!enable)

next_state= idle;

else next_state=busy;

end

error:

begin

haddr= 0 ;

next_state= idle;

end

default:

begin

hsel= 2'bxx;

end

endcase

end

endmodule

Slave

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Design Name:

// Module Name: ahb_slave

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

/////////////// This is the code for the SLAVE //////////////////////////////////

/////////////// Working and code is as per the slave state model ///////////////

`define NON_SEQ     2'b10

`define SEQ 	    2'b11

`define BUSY 	    2'b01

`define IDLE        2'b00

`define OKAY  1'b0

`define ERROR 1'b1

//`define RETRY 2'b10

//`define SPLIT 2'b11

module ahb_slave(

input clk,

input [31:0] hwdata,

input [31:0] haddr,

input [2:0] hsize,

input [2:0] hburst,

input hresetn, hsel, hwrite,

//input hreadyout,

input [1:0] htrans,

output reg hresp,

output reg hready,

output reg [31:0] hrdata

);

logic [7:0] mem[256] = '{default : 10};

//////////////////////////// Function for single transaction //////////////////////////////////////////////

function bit[31:0] single_tr (input bit [31:0] addr, input bit [2:0] hsize);

unique case(hsize)

3'b000: begin

mem[addr]  = hwdata[7:0];

end

3'b001: begin

mem[addr]     = hwdata[7:0];

mem[addr + 1] = hwdata[15:8];

end

3'b010: begin

mem[addr]     = hwdata[7:0];

mem[addr + 1] = hwdata[15:8];

mem[addr + 2] = hwdata[23:16];

mem[addr + 3] = hwdata[31:24];

end

default: mem[addr]  = 8'bxxxxxxxx;

endcase

return addr;

endfunction

////////////////////////// Function for unspecified increment mode /////////////////////////////////////////////////////////

function bit[31:0] unincr_wr (input bit [31:0] addr, input bit [2:0] hsize);

bit [31:0] raddr;

unique case(hsize)

3'b000: begin

mem[addr]    = hwdata[7:0];

raddr        = addr + 1;

end

3'b001: begin

mem[addr]     = hwdata[7:0];

mem[addr + 1] = hwdata[15:8];

raddr         = addr + 2;

end

3'b010: begin

mem[addr]     = hwdata[7:0];

mem[addr + 1] = hwdata[15:8];

mem[addr + 2] = hwdata[23:16];

mem[addr + 3] = hwdata[31:24];

raddr         = addr + 4;

end

default: mem[addr]  = 8'bxxxxxxxx;

endcase

return raddr;

endfunction

////////////////////// Function for defining the boundary when slave in wrap mode , defines the boundary/length of data ////////////////////////////////////////////

function bit[7:0] boundary(input bit [2:0] hburst, input [2:0] hsize);

bit [7:0] temp;

unique case(hsize)

3'b000: begin

unique case (hburst)

3'b010 : temp = 4 * 1;

3'b100 : temp =  8 * 1;

3'b110 : temp = 16 * 1;

default: temp = 0;

endcase

end

3'b001 : begin

unique case (hburst)

3'b010 :  temp = 4 * 2;

3'b100 : temp =  8 * 2;

3'b110 : temp = 16 * 2;

default: temp = 0;

endcase

end

3'b010 : begin

unique case (hburst)

3'b010 :  temp = 4 * 4;

3'b100 : temp =  8 * 4;

3'b110 : temp = 16 * 4;

default: temp = 0;

endcase

end

endcase

return temp;

endfunction

//////////////////// Function for wrap mode, uses the boundary value generated by the boundary function ///////////////////////////////////////////////////////

function bit[31:0] wrap_wr (input bit [31:0] addr, input bit [7:0] boundary, input [2:0] hsize);

bit [31:0] addr1, addr2, addr3,addr4;

unique case(hsize)

3'b000: begin

mem[addr] = hwdata[7:0];

if((addr + 1) % boundary == 0)

addr1 = (addr + 1) - boundary;

else

addr1 = (addr + 1);

return addr1;

end

3'b001: begin

mem[addr] = hwdata[7:0];

if((addr + 1) % boundary == 0)

addr1 = (addr + 1) - boundary;

else

addr1 = (addr + 1);

mem[addr1] = hwdata[15:8];

if((addr1 + 1) % boundary == 0)

addr2 = (addr1 + 1) - boundary;

else

addr2 = (addr1 + 1);

return addr2;

end

3'b010: begin

mem[addr] = hwdata[7:0];

if((addr + 1) % boundary == 0)

addr1 = (addr + 1) - boundary;

else

addr1 = (addr + 1);

mem[addr1] = hwdata[15:8];

if((addr1 + 1) % boundary == 0)

addr2 = (addr1 + 1) - boundary;

else

addr2 = (addr1 + 1);

mem[addr2] = hwdata[23:16];

if((addr2 + 1) % boundary == 0)

addr3 = (addr2 + 1) - boundary;

else

addr3 = (addr2 + 1);

mem[addr3] = hwdata[31:24];

if((addr3 + 1) % boundary == 0)

addr4 = (addr3 + 1) - boundary;

else

addr4 = (addr3 + 1);

return addr4;

end

default: return addr;

endcase

endfunction

////////////////////////// Function for increment mode, works on hsize /////////////////////////////////////////////////////////

function bit[31:0] incr_wr(input bit [31:0] addr, input bit [2:0] hsize);

bit [31:0] raddr;

unique case(hsize)

3'b000: begin

mem[addr]    = hwdata[7:0];

raddr        = addr + 1;

end

3'b001: begin

mem[addr]     = hwdata[7:0];

mem[addr + 1] = hwdata[15:8];

raddr         = addr + 2;

end

3'b010: begin

mem[addr]     = hwdata[7:0];

mem[addr + 1] = hwdata[15:8];

mem[addr + 2] = hwdata[23:16];

mem[addr + 3] = hwdata[31:24];

raddr         = addr + 4;

end

default: mem[addr]  = 8'bxxxxxxxx;

endcase

return raddr;

endfunction

////////////////////////////  Single transfer read ////////////////////////////////////////////////////////////////////////////////

function bit[31:0] single_tr_rd (input bit [31:0] addr, input bit [2:0] hsize);

unique case(hsize)

3'b000: begin

hrdata[7:0] = mem[addr];

end

3'b001: begin

hrdata[7:0]  = mem[addr];

hrdata[15:8] = mem[addr + 1];

end

3'b010: begin

hrdata[7:0]   = mem[addr];

hrdata[15:8]  = mem[addr + 1];

hrdata[23:16] = mem[addr + 2];

hrdata[31:24] = mem[addr + 3];

end

default: hrdata[7:0]  = 8'bxxxxxxxx;

endcase

return addr;

endfunction

///////////////////////////// Read for unspecified length ///////////////////////////////////////////////////////

function bit[31:0] unincr_rd (input bit [31:0] addr, input bit [2:0] hsize);

bit [31:0] raddr;

unique case(hsize)

3'b000: begin

hrdata[7:0] = mem[addr];

raddr        = addr + 1;

end

3'b001: begin

hrdata[7:0] = mem[addr];

hrdata[15:8] = mem[addr + 1];

raddr         = addr + 2;

end

3'b010: begin

hrdata[7:0]   = mem[addr];

hrdata[15:8]  = mem[addr + 1];

hrdata[23:16] = mem[addr + 2];

hrdata[31:24] = mem[addr + 3];

raddr         = addr + 4;

end

default: begin

hrdata[7:0]  = 8'bxxxxxxxx;

raddr = addr;

end

endcase

return raddr;

endfunction

///////////////////////////// Wrapping Read, uses the Boundary length ////////////////////////////////////////////////////

function bit[31:0] wrap_rd (input bit [31:0] addr, input bit [7:0] boundary, input [2:0] hsize);

bit [31:0] addr1, addr2, addr3,addr4;

unique case(hsize)

3'b000: begin

hrdata[7:0] = mem[addr];

if((addr + 1) % boundary == 0)

addr1 = (addr + 1) - boundary;

else

addr1 = (addr + 1);

return addr1;

end

3'b001: begin

hrdata[7:0] = mem[addr];

if((addr + 1) % boundary == 0)

addr1 = (addr + 1) - boundary;

else

addr1 = (addr + 1);

hrdata[15:8] = mem[addr1];

if((addr1 + 1) % boundary == 0)

addr2 = (addr1 + 1) - boundary;

else

addr2 = (addr1 + 1);

return addr2;

end

3'b010: begin

hrdata[7:0] = mem[addr];

if((addr + 1) % boundary == 0)

addr1 = (addr + 1) - boundary;

else

addr1 = (addr + 1);

hrdata[15:8] = mem[addr1];

if((addr1 + 1) % boundary == 0)

addr2 = (addr1 + 1) - boundary;

else

addr2 = (addr1 + 1);

hrdata[23:16] = mem[addr2];

if((addr2 + 1) % boundary == 0)

addr3 = (addr2 + 1) - boundary;

else

addr3 = (addr2 + 1);

hrdata[31:24] = mem[addr3];

if((addr3 + 1) % boundary == 0)

addr4 = (addr3 + 1) - boundary;

else

addr4 = (addr3 + 1);

return addr4;

end

default: return addr;

endcase

endfunction

//////////////////////////////  Increment mode Read ///////////////////////////////////////////////////////////

function bit[31:0] incr_rd(input bit [31:0] addr, input bit [2:0] hsize);

bit [31:0] raddr;

unique case(hsize)

3'b000: begin

hrdata[7:0]  = mem[addr];

raddr        = addr + 1;

end

3'b001: begin

hrdata[7:0]  = mem[addr];

hrdata[15:8] = mem[addr + 1];

raddr         = addr + 2;

end

3'b010: begin

hrdata[7:0]   = mem[addr];

hrdata[15:8]  = mem[addr + 1];

hrdata[23:16] = mem[addr + 2];

hrdata[31:24] = mem[addr + 3];

raddr         = addr + 4;

end

default: hrdata[7:0]  = 8'bxxxxxxxx;

endcase

return raddr;

endfunction

//////////////////////// Defining the state modes ////////////////////////////////////

typedef enum  {idle = 0, check_mode = 1, write = 2, read = 3} state_type;

state_type slave_state, next_state;

///////////////////////////////// Below starts the FSM working //////////////////////////////////////////////

//////////////////////////////////\\\\\\\\\ CHANGING STATE AT EVERY CLOCK \\\\\\\\/////////////////////////////////////////////////

always_ff@(posedge clk)

begin

if(!hresetn)

slave_state <= idle;

else

slave_state <= next_state;

end

integer len_count = 0;

reg [31:0] retaddr;

reg [31:0] next_addr;

reg [7:0] wboundary;

always_comb

begin

case(slave_state)

idle :

begin

if (hsel) begin

next_state = check_mode;

hready = 1'b0;

len_count = 0;

end

else next_state= idle;

end

////////////////////--------------------------------------- WE ARE IN CHECK MODE ---------------------------//////////////////////////////////////////////

check_mode :

begin

hready = 1'b0;

if(hresetn && hsel)  ///hreadyout

begin

if(haddr>255)

begin

hresp=`ERROR;

next_state= idle;

end

else if(htrans == `NON_SEQ)

begin

next_addr = haddr;

hresp=`OKAY;

if(hwrite)

next_state = write;

else

next_state = read;

end

else if (htrans == `SEQ)

begin

next_addr  = retaddr;

hresp=`OKAY;

if(hwrite)

next_state = write;

else

next_state = read;

end

else next_state=idle;

end

else begin

hresp=`OKAY;

next_state=idle;

end

end

////////////////////--------------------------------------- WE ARE IN WRITE MODE ---------------------------//////////////////////////////////////////////

write:

begin

case(hburst)

//////////////////////////Single Write at HADDR //////////////////////////////

3'b000: begin

retaddr = single_tr(next_addr,hsize);

hready     = 1'b1;

next_state = idle;

hresp = `OKAY;

end

/////////////////////Increment for Unspecified Length ////////////////////////////

3'b001:

begin   ////incr mode

if(!hwrite) begin

next_state=idle;

end

else begin

hready = 1'b1;

retaddr = unincr_wr(next_addr, hsize);

hresp = `OKAY;

if(len_count < 32)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

len_count = 0;

next_state = idle;

end

end

end

//////////////////////////// 4 beat wrapping ////////////////////////////////////////////

3'b010:

begin

hready = 1'b1;

wboundary = boundary(hburst, hsize);

retaddr   = wrap_wr(next_addr, wboundary, hsize);

hresp = `OKAY;

if(len_count <= 2) // 0 1 2 3

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

///////////////////////////////  4 beat Incrementing /////////////////////////////////////////

3'b011:

begin

hready = 1'b1;

retaddr = incr_wr(next_addr, hsize);

hresp = `OKAY;

if(len_count <= 2)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

//////////////////////////////////////////////// 8 beat wrapping //////////////////////////////////////////////

3'b100:

begin

hready = 1'b1;

wboundary = boundary(hburst, hsize);

retaddr = wrap_wr(next_addr, wboundary, hsize);

hresp = `OKAY;

if(len_count <= 6)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

///////////////////////////////////////// 8 beat Incrementing ////////////////////////////////////////

3'b101:

begin

hready = 1'b1;

retaddr = incr_wr(next_addr, hsize);

hresp = `OKAY;

if(len_count <= 6)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

///////////////////////////////////////////////// 16 beat wrapping ///////////////////////////////////////////

3'b110:

begin

hready = 1'b1;

wboundary = boundary(hburst, hsize);

retaddr = wrap_wr(next_addr, wboundary, hsize);

hresp = `OKAY;

if(len_count <= 14)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

/////////////////////////////////////////// 16 beat increment ///////////////////////////////////////

3'b111:

begin

hready = 1'b1;

retaddr = incr_wr(next_addr, hsize);

hresp = `OKAY;

if(len_count <= 14)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

endcase

end

////////////////////--------------------------------------- WE ARE IN READ MODE ---------------------------//////////////////////////////////////////////

read : begin

case(hburst)

///////////////////////////////////////////// Single Write at HADDR ///////////////////////////////////////////

3'b000: begin  ////single transfer

retaddr = single_tr_rd(haddr,hsize);

hready     = 1'b1;

next_state = idle;

hresp = `OKAY;

end

/////////////////////////////////////// INCREMENT for UNSPECIFIED LENGTH ////////////////////////////////////////////////////////

3'b001:

begin   ////incr mode

if(hwrite) begin

next_state=idle;

end

else begin

hready = 1'b1;

retaddr = unincr_rd(next_addr, hsize);

hresp = `OKAY;

if( len_count < 32)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

len_count = 0;

next_state = idle;

end

end

end

/////////////////////////////////////////////////////// 4 beat wrapping //////////////////////////////////////////////////////

3'b010:

begin

hready = 1'b1;

wboundary = boundary(hburst, hsize);

retaddr = wrap_rd(next_addr, wboundary, hsize);

hresp = `OKAY;

if(len_count <= 2)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

/////////////////////////////// 4 beat Incrementing read /////////////////////////////////////////////////////////

3'b011:

begin

hready = 1'b1;

retaddr = incr_rd(next_addr, hsize);

hresp = `OKAY;

if(len_count <= 2)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

//////////////////////////////////////////////// 8 beat wrapping ///////////////////////////////////////////////////////////

3'b100:

begin

hready = 1'b1;

wboundary = boundary(hburst, hsize);

retaddr = wrap_rd(next_addr, wboundary, hsize);

hresp = `OKAY;

if(len_count <= 6)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

///////////////////////////////////////// 8 beat Incrementing ///////////////////////////////////////////////////

3'b101:

begin

hready = 1'b1;

retaddr = incr_rd(next_addr, hsize);

hresp = `OKAY;

if(len_count <= 6)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

///////////////////////////////////////////////// 16 beat wrapping /////////////////////////////////////////////////////////

3'b110:

begin

hready = 1'b1;

wboundary = boundary(hburst, hsize);

retaddr = wrap_rd(next_addr, wboundary, hsize);

hresp = `OKAY;

if(len_count <= 14)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

/////////////////////////////////////////////////// 16 beat increment ///////////////////////////////////

3'b111:

begin

hready = 1'b1;

retaddr = incr_rd(next_addr, hsize);

hresp = `OKAY;

if(len_count <= 14)

begin

len_count = len_count + 1;

next_state = check_mode;

end

else

begin

next_state = idle;

len_count = 0;

end

end

endcase

end

endcase

end

endmodule

Slave Testbench

`timescale 1ns/1ns

`define NON_SEQ     2'b10

`define SEQ 	    2'b11

`define BUSY 	    2'b01

`define IDLE        2'b00

`define OKAY  1'b0

`define ERROR 1'b1

module ahb_slave_tb();

reg hclk;

reg hresetn;

reg hsel;

reg [31:0] haddr;

reg hwrite;

reg [2:0] hsize;

reg [2:0] hburst;

//reg [3:0] hprot;

reg [1:0] htrans;

//reg hmastlock;

reg hready;

reg [31:0] hwdata;

//wire hreadyout;

wire hresp;

wire [31:0] hrdata;

initial begin

hclk = 0;

hresetn = 1;

hsel = 0;

haddr = 32'd0;

hwrite = 1;

hburst = 3'b001;

// hprot = 0;

hsize = 0;

/////////////////////////////////

htrans = `NON_SEQ;

////////////////////////////////////

// hmastlock = 0;

//  hready = 1;

hwdata = 32'd0;

#10 $monitor("-------------------everything reset---------------------");

hresetn = 0;

#10 hresetn = 1;

//  repeat(2)

//  @(posedge hclk);

//   hsel = 1;

//  haddr = 0;

//  hwrite = 1'b1;

//  hwdata = 8;

//  hready = 1;

//#20 ;

//  repeat(2)

//  @(posedge hclk);

//  hsel = 1;

// // haddr = addr;

// haddr = 0;

//  hwrite = 1'b0;

//  hready = 1;

// burst: incr

//burst_incr_wr();

// basic write and read

// basic_wr();

// Wait state write and read

//wr_wait_state();

// burst: incr

//burst_incr_wr();

// burst wrap 4

burst_wrap4(32'd1);

// burst incr 4

// burst_incr4();

#80 ;

$finish;

end

always #2 hclk <= ~hclk;

//----------------------------------------------------

// Normal write and read

//----------------------------------------------------

task write(input [31:0] addr, input [31:0] data);

begin

repeat(2)

@(posedge hclk)

hsel = 1;

haddr = addr;

hwrite = 1'b1;

hwdata = data;

hready = 1;

//@(posedge hclk)

//  hsel = 0;

end

endtask

task read(input [31:0] addr);

begin

repeat(2)

@(posedge hclk)

hsel = 1;

haddr = addr;

//haddr = 0;

hwrite = 1'b0;

hready = 1;

// @(posedge hclk)

// hsel = 0;

end

endtask

//----------------------------------------------------

// Burst

//----------------------------------------------------

// Incr

task burst_incr_wr();

begin

hburst = 3'b001;

hsel= 1;

htrans = `NON_SEQ;

write(32'd1,32'd1);

htrans = `SEQ;

write(32'd1,32'd2);

write(32'd1,32'd3);

write(32'd1,32'd4);

hburst = 3'b000;

//   repeat(3)

//  @(posedge hclk)

hsel=0;

#4

hsel=1;

htrans = `NON_SEQ;

hburst = 3'b001;

read(32'd1);

htrans = `SEQ;

read(32'd1);

read(32'd1);

read(32'd1);

hburst = 3'b000;

end

endtask

//Wrap 4

task burst_wrap4(input [31:0] addr);

begin

//  // write

htrans = `NON_SEQ;

@(posedge hclk);

write(32'd1,32'd1);

hburst = 3'b010;

// htrans = `NON_SEQ;

// write(32'd1,32'd1);

htrans = `SEQ;

@(posedge hclk);

write(32'd1,32'd2);

write(32'd1,32'd3);

write(32'd1,32'd4);

write(32'd1,32'd5);

write(32'd1,32'd6);

write(32'd1,32'd7);

write(32'd1,32'd8);

write(32'd1,32'd9);

write(32'd1,32'd10);

//@(posedge hclk);

htrans = `NON_SEQ;

@(posedge hclk);

read(32'd1);

hburst = 3'b010;

htrans = `SEQ;

@(posedge hclk);

read(32'd1);

read(32'd1);

read(32'd1);

read(32'd1);

read(32'd1);

read(32'd1);

read(32'd1);

read(32'd1);

read(32'd1);

//  repeat(2)

//  @(posedge hclk)

//    hburst = 3'b010;

//  hsel = 1'b1;

//  hwrite = 1'b1;

//  haddr = addr;

//  hready = 1'b1;

//  @(posedge hclk)

//  hwdata = 32'd1;

//  @(posedge hclk)

//  hwdata = 32'd2;

//  @(posedge hclk)

//  hwdata = 32'd3;

//  @(posedge hclk)

//  hwdata = 32'd4;

//  @(posedge hclk)

//  hwdata = 32'd5;

//  @(posedge hclk)

//  hwdata = 32'd6;

//  @(posedge hclk)

//  hsel = 1'b0;

//  hwrite = 1'b0;

//  hburst = 3'b000;

//  // read

//  @(posedge hclk)

//  hburst = 3'b010;

//  hsel = 1'b1;

//  hwrite = 1'b0;

//  haddr = addr;

//  hready = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  hburst = 3'b000;

end

endtask

//// Incr 4

//task burst_incr4();

//begin

//  // write

//  @(posedge hclk)

//  hburst = 3'b011;

//  hsel = 1'b1;

//  hwrite = 1'b1;

//  haddr = 32'd1;

//  hready = 1'b1;

//  @(posedge hclk)

//  hwdata = 32'd1;

//  @(posedge hclk)

//  hwdata = 32'd2;

//  @(posedge hclk)

//  hwdata = 32'd3;

//  @(posedge hclk)

//  hwdata = 32'd4;

//  @(posedge hclk)

//  hsel = 1'b0;

//  hwrite = 1'b0;

//  hburst = 3'b000;

//  // read

//  @(posedge hclk)

//  hburst = 3'b011;

//  hsel = 1'b1;

//  hwrite = 1'b0;

//  haddr = 32'd1;

//  hready = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  @(posedge hclk)

//  hsel = 1'b1;

//  @(posedge hclk)

//  hsel = 1'b0;

//  hburst = 3'b000;

//end

//endtask

////----------------------------------------------------

//// Basic

////----------------------------------------------------

//task basic_wr();

//begin

//  hburst = 3'b000;

//  write(32'd0,32'd100);

//  repeat(6)

//  @(posedge hclk)

//  read(32'd0);

//  repeat(6)

//  @(posedge hclk)

//  ;

////  write(32'd1,32'd1);

////  read(32'd1);

////  write(32'd2,32'd2);

////  read(32'd2);

////  write(32'd3,32'd3);

////  read(32'd3);

////  write(32'd4,32'd4);

////  write(32'd5,32'd5);

////  write(32'd6,32'd6);

////  read(32'd4);

////  read(32'd5);

////  read(32'd6);

//end

//endtask

//task wr_wait_state();

//begin

//  hburst = 3'b000;

//  write_wait_state(32'd7,32'd7);

//  read_wait_state(32'd7);

//  write_wait_state(32'd8,32'd8);

//  read_wait_state(32'd8);

//  read_wait_state(32'd7);

//end

//endtask

//task write_wait_state(input [31:0] addr, input [31:0] data);

//begin

//  @(posedge hclk)

//  hsel = 1;

//  haddr = addr;

//  hwrite = 1'b1;

//  hwdata = data;

//  hready = 0;

//  @(posedge hclk)

//  hready = 0;

//  @(posedge hclk)

//  hready = 1;

//  @(posedge hclk)

//  hsel = 0;

//end

//endtask

//task read_wait_state(input [31:0] addr);

//begin

//  @(posedge hclk)

//  hsel = 1;

//  haddr = addr;

//  hwrite = 1'b0;

//  hready = 0;

//  @(posedge hclk)

//  hready = 0;

//  @(posedge hclk)

//  hready = 1;

//  @(posedge hclk)

//  hsel = 0;

//end

//endtask

ahb_slave uut(

.clk(hclk),

.hresetn(hresetn),

.hsel(hsel),

.haddr(haddr),

.hwrite(hwrite),

.hsize(hsize),

.hburst(hburst),

// .hprot(hprot),

.htrans(htrans),

// .hmastlock(hmastlock),

.hready(hready),

.hwdata(hwdata),

// .hreadyout(hreadyout),

.hresp(hresp),

.hrdata(hrdata)

);

Endmodule

Top

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Design Name:

// Module Name: top_ahb

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

///////////////////////////// BELOW IS THE TOP LEVEL DESIGN WHICH INTEGRATES ALL THE MODULES, THE CONNECTIONS ARE DONE AS PER THE DIAGRAM IN THE DOCS FOLDER ///////////////////////////

module top_ahb(

input clk,

input hresetn,

input enable,

input [31:0] in_hwdata,

input [31:0] in_haddr,

input [2:0] in_hsize,

input [2:0] in_hburst,

input [1:0] in_hsel,

input in_hwrite,

input [1:0] in_htrans,

//output logic [31:0] hwdata,

//output logic [31:0] haddr,

//output logic [2:0] hsize,

//output logic [2:0] hburst,

//output logic [1:0]hsel,

//output logic hwrite,

//output logic [1:0] htrans,

output [31:0] out_hrdata

);

wire hresp;

wire hresp1;

wire hresp2;

wire hresp3;

wire hresp4;

wire hready;

wire hready1;

wire hready2;

wire hready3;

wire hready4;

wire [31:0] hrdata;

wire [31:0] hrdata1;

wire [31:0] hrdata2;

wire [31:0] hrdata3;

wire [31:0] hrdata4;

wire hsel1;

wire hsel2;

wire hsel3;

wire hsel4;

wire [31:0] hwdata;

wire [31:0] haddr;

wire [2:0] hsize;

wire [2:0] hburst;

wire [1:0]hsel;

wire hwrite;

wire [1:0] htrans;

// Master

ahb_master master(

.clk(clk),

.hresetn(hresetn),

.enable(enable),

.hresp(hresp),

.hready(hready),

.hrdata(hrdata),

.in_hwdata(in_hwdata),

.in_haddr(in_haddr),

.in_hsize(in_hsize),

.in_hburst(in_hburst),

.in_hsel(in_hsel),

.in_hwrite(in_hwrite),

.in_htrans(in_htrans),

.hwdata(hwdata),

.haddr(haddr),

.hsize(hsize),

.hburst(hburst),

.hsel(hsel),

.hwrite(hwrite),

.htrans(htrans),

.out_hrdata(out_hrdata)

);

// Mux

mux mux1(

.hrdata_1(hrdata1),

.hrdata_2(hrdata2),

.hrdata_3(hrdata3),

.hrdata_4(hrdata4),

.hready_1(hready1),

.hready_2(hready2),

.hready_3(hready3),

.hready_4(hready4),

.hresp_1(hresp1),

.hresp_2(hresp2),

.hresp_3(hresp3),

.hresp_4(hresp4),

.sel(hsel),

.hrdata(hrdata),

.hready(hready),

.hresp(hresp)

);

// Decoder

decoder decoder1(

.hsel(hsel),

.hsel_1(hsel1),

.hsel_2(hsel2),

.hsel_3(hsel3),

.hsel_4(hsel4)

);

// Slave 1

ahb_slave slave_1(

.clk(clk),

.hwdata(hwdata),

.haddr(haddr),

.hsize(hsize),

.hburst(hburst),

.hresetn(hresetn),

.hsel(hsel1),

.hwrite(hwrite),

//.hreadyout()

.htrans(htrans),

.hresp(hresp1),

.hready(hready1),

.hrdata(hrdata1)

);

// Slave 2

ahb_slave slave_2(

.clk(clk),

.hwdata(hwdata),

.haddr(haddr),

.hsize(hsize),

.hburst(hburst),

.hresetn(hresetn),

.hsel(hsel2),

.hwrite(hwrite),

//.hreadyout()

.htrans(htrans),

.hresp(hresp2),

.hready(hready2),

.hrdata(hrdata2)

);

// Slave 3

ahb_slave slave_3(

.clk(clk),

.hwdata(hwdata),

.haddr(haddr),

.hsize(hsize),

.hburst(hburst),

.hresetn(hresetn),

.hsel(hsel3),

.hwrite(hwrite),

//.hreadyout()

.htrans(htrans),

.hresp(hresp3),

.hready(hready3),

.hrdata(hrdata3)

);

// Slave 4

ahb_slave slave_4(

.clk(clk),

.hwdata(hwdata),

.haddr(haddr),

.hsize(hsize),

.hburst(hburst),

.hresetn(hresetn),

.hsel(hsel4),

.hwrite(hwrite),

//.hreadyout()

.htrans(htrans),

.hresp(hresp4),

.hready(hready4),

.hrdata(hrdata4)

);

Endmodule

# Waveform

# Result 1: Single transaction write and read

# Result 2: 4 beat w

# Result 3: 4 beat increment write and r

# Result 4: Unspecified increment write and read