Lab 4 – Week 4

DMA Protocol

Design Code

`timescale 1ns / 1ps

module axi_dma_controller #(

parameter int ADDR_WD = 32,

parameter int DATA_WD = 32,

parameter int CHANNEL_COUNT = 8, // 8 channels

parameter int MAX_BURST_LEN = 16, // 16 beats

localparam int STRB_WD = DATA_WD / 8

) (

input wire                    clk,

input wire                    rst,

// DMA Command

input wire                    cmd_valid,

input wire [ADDR_WD-1:0]      cmd_src_addr,

input wire [ADDR_WD-1:0]      cmd_dst_addr,

input wire [1:0]              cmd_burst,

input wire [ADDR_WD-1:0]      cmd_len,

input wire [2:0]              cmd_size,

output wire                   cmd_ready,

// Read Address Channel

output wire                   M_AXI_ARVALID,

output wire [ADDR_WD-1:0]     M_AXI_ARADDR,

output wire [7:0]             M_AXI_ARLEN,

output wire [2:0]             M_AXI_ARSIZE,

output wire [1:0]             M_AXI_ARBURST,

input wire                    M_AXI_ARREADY,

// Read Response Channel

input wire                    M_AXI_RVALID,

input wire [DATA_WD-1:0]      M_AXI_RDATA,

input wire [1:0]              M_AXI_RRESP,

input wire                    M_AXI_RLAST,

output wire                   M_AXI_RREADY,

// Write Address Channel

output wire                   M_AXI_AWVALID,

output wire [ADDR_WD-1:0]     M_AXI_AWADDR,

output wire [7:0]             M_AXI_AWLEN,

output wire [2:0]             M_AXI_AWSIZE,

output wire [1:0]             M_AXI_AWBURST,

input wire                    M_AXI_AWREADY,

// Write Data Channel

output wire                   M_AXI_WVALID,

output wire [DATA_WD-1:0]     M_AXI_WDATA,

output wire [STRB_WD-1:0]     M_AXI_WSTRB,

output wire                   M_AXI_WLAST,

input wire                    M_AXI_WREADY,

// Write Response Channel

input wire                    M_AXI_BVALID,

input wire [1:0]              M_AXI_BRESP,

output wire                   M_AXI_BREADY

);

logic                             buf_cmd_in_valid;

logic                             buf_cmd_in_ready;

logic [$clog2(ADDR_WD/8)-1:0]     buf_cmd_in_src_offset;

logic [ADDR_WD-1:0]               buf_cmd_in_dst_addr;

logic [1:0]                       buf_cmd_in_burst;

logic [ADDR_WD-1:0]               buf_cmd_in_len;

logic [2:0]                       buf_cmd_in_size;

logic                             buf_cmd_out_valid;

logic                             buf_cmd_out_ready;

logic [$clog2(ADDR_WD/8)-1:0]     buf_cmd_out_src_offset;

logic [ADDR_WD-1:0]               buf_cmd_out_dst_addr;

logic [1:0]                       buf_cmd_out_burst;

logic [ADDR_WD-1:0]               buf_cmd_out_len;

logic [2:0]                       buf_cmd_out_size;

logic                             buf_data_in_valid;

logic                             buf_data_in_ready;

logic [DATA_WD-1:0]               buf_data_in;

logic                             buf_data_in_last;

logic                             buf_data_out_valid;

logic                             buf_data_out_ready;

logic [DATA_WD-1:0]               buf_data_out;

logic                             buf_data_out_last;

dmac_read #(

.ADDR_WD(ADDR_WD),

.DATA_WD(DATA_WD),

.CHANNEL_COUNT(CHANNEL_COUNT), // 8 channels

.MAX_BURST_LEN(MAX_BURST_LEN)  // 16 beats

) i_rd (

.clk,

.rst,

// DMA Command

.cmd_valid,

.cmd_ready,

.cmd_src_addr,

.cmd_dst_addr,

.cmd_burst,

.cmd_len,

.cmd_size,

.cmd_out_valid(buf_cmd_in_valid),

.cmd_out_ready(buf_cmd_in_ready),

.cmd_out_src_offset(buf_cmd_in_src_offset),

.cmd_out_dst_addr(buf_cmd_in_dst_addr),

.cmd_out_burst(buf_cmd_in_burst),

.cmd_out_len(buf_cmd_in_len),

.cmd_out_size(buf_cmd_in_size),

.data_out_valid(buf_data_in_valid),

.data_out_ready(buf_data_in_ready),

.data_out(buf_data_in),

.data_out_last(buf_data_in_last),

// Read Address Channel

.m_axi_arvalid(M_AXI_ARVALID),

.m_axi_araddr(M_AXI_ARADDR),

.m_axi_arlen(M_AXI_ARLEN),

.m_axi_arsize(M_AXI_ARSIZE),

.m_axi_arburst(M_AXI_ARBURST),

.m_axi_arready(M_AXI_ARREADY),

// Read Response Channel

.m_axi_rvalid(M_AXI_RVALID),

.m_axi_rdata(M_AXI_RDATA),

.m_axi_rresp(M_AXI_RRESP),

.m_axi_rlast(M_AXI_RLAST),

.m_axi_rready(M_AXI_RREADY)

);

dmac_buffer #(

.ADDR_WD(ADDR_WD),

.DATA_WD(DATA_WD),

.CHANNEL_COUNT(CHANNEL_COUNT)

) i_buf(

.clk,

.rst,

.cmd_in_valid(buf_cmd_in_valid),

.cmd_in_ready(buf_cmd_in_ready),

.cmd_in_src_offset(buf_cmd_in_src_offset),

.cmd_in_dst_addr(buf_cmd_in_dst_addr),

.cmd_in_burst(buf_cmd_in_burst),

.cmd_in_len(buf_cmd_in_len),

.cmd_in_size(buf_cmd_in_size),

.data_in_valid(buf_data_in_valid),

.data_in_ready(buf_data_in_ready),

.data_in(buf_data_in),

.data_in_last(buf_data_in_last),

.cmd_out_valid(buf_cmd_out_valid),

.cmd_out_ready(buf_cmd_out_ready),

.cmd_out_src_offset(buf_cmd_out_src_offset),

.cmd_out_dst_addr(buf_cmd_out_dst_addr),

.cmd_out_burst(buf_cmd_out_burst),

.cmd_out_len(buf_cmd_out_len),

.cmd_out_size(buf_cmd_out_size),

.data_out_valid(buf_data_out_valid),

.data_out_ready(buf_data_out_ready),

.data_out(buf_data_out),

.data_out_last(buf_data_out_last)

);

dmac_write #(

.ADDR_WD(ADDR_WD),

.DATA_WD(DATA_WD),

.CHANNEL_COUNT(CHANNEL_COUNT), // 8 channels

.MAX_BURST_LEN(MAX_BURST_LEN)  // 16 beats

) i_wr (

.clk,

.rst,

.cmd_in_valid(buf_cmd_out_valid),

.cmd_in_ready(buf_cmd_out_ready),

.cmd_in_src_offset(buf_cmd_out_src_offset),

.cmd_in_dst_addr(buf_cmd_out_dst_addr),

.cmd_in_burst(buf_cmd_out_burst),

.cmd_in_len(buf_cmd_out_len),

.cmd_in_size(buf_cmd_out_size),

.data_in_valid(buf_data_out_valid),

.data_in_ready(buf_data_out_ready),

.data_in(buf_data_out),

.data_in_last(buf_data_out_last),

// Write Address Channel

.m_axi_awvalid(M_AXI_AWVALID),

.m_axi_awaddr(M_AXI_AWADDR),

.m_axi_awlen(M_AXI_AWLEN),

.m_axi_awsize(M_AXI_AWSIZE),

.m_axi_awburst(M_AXI_AWBURST),

.m_axi_awready(M_AXI_AWREADY),

// Write Data Channel

.m_axi_wvalid(M_AXI_WVALID),

.m_axi_wdata(M_AXI_WDATA),

.m_axi_wstrb(M_AXI_WSTRB),

.m_axi_wlast(M_AXI_WLAST),

.m_axi_wready(M_AXI_WREADY),

// Write Response Channel

.m_axi_bvalid(M_AXI_BVALID),

.m_axi_bresp(M_AXI_BRESP),

.m_axi_bready(M_AXI_BREADY)

);

// Wrapped in initial block to prevent Vivado VRFC 10-2377 elaboration errors

initial begin

if (ADDR_WD % 8 != 0) $error("ADDR_WD is not multiple of 8");

if (DATA_WD % 8 != 0) $error("DATA_WD is not multiple of 8");

if (2 ** $clog2(ADDR_WD / 8) != ADDR_WD / 8) $error("ADDR_WD is not power of 2");

if (2 ** $clog2(DATA_WD / 8) != DATA_WD / 8) $error("DATA_WD is not power of 2");

if (2 ** $clog2(MAX_BURST_LEN) != MAX_BURST_LEN) $error("MAX_BURST_LEN is not power of 2");

if (MAX_BURST_LEN <= 0 || MAX_BURST_LEN > 256)   $error("MAX_BURST_LEN is invalid");

end

endmodule : axi_dma_controller

Testbench

`timescale 1ns/1ps

module tb_axi_dma_controller;

// Parameters

parameter int ADDR_WD = 32;

parameter int DATA_WD = 32;

parameter int CHANNEL_COUNT = 8;

parameter int MAX_BURST_LEN = 16;

localparam int STRB_WD = DATA_WD / 8;

// Clock and Reset

logic clk;

logic rst;

// DMA Command

logic                 cmd_valid;

logic [ADDR_WD-1:0]   cmd_src_addr;

logic [ADDR_WD-1:0]   cmd_dst_addr;

logic [1:0]           cmd_burst;

logic [ADDR_WD-1:0]   cmd_len;

logic [2:0]           cmd_size;

wire                  cmd_ready;

// AXI Master Interfaces

wire                  M_AXI_ARVALID;

wire  [ADDR_WD-1:0]   M_AXI_ARADDR;

wire  [7:0]           M_AXI_ARLEN;

wire  [2:0]           M_AXI_ARSIZE;

wire  [1:0]           M_AXI_ARBURST;

logic                 M_AXI_ARREADY;

logic                 M_AXI_RVALID;

logic [DATA_WD-1:0]   M_AXI_RDATA;

logic [1:0]           M_AXI_RRESP;

logic                 M_AXI_RLAST;

wire                  M_AXI_RREADY;

wire                  M_AXI_AWVALID;

wire  [ADDR_WD-1:0]   M_AXI_AWADDR;

wire  [7:0]           M_AXI_AWLEN;

wire  [2:0]           M_AXI_AWSIZE;

wire  [1:0]           M_AXI_AWBURST;

logic                 M_AXI_AWREADY;

wire                  M_AXI_WVALID;

wire  [DATA_WD-1:0]   M_AXI_WDATA;

wire  [STRB_WD-1:0]   M_AXI_WSTRB;

wire                  M_AXI_WLAST;

logic                 M_AXI_WREADY;

logic                 M_AXI_BVALID;

logic [1:0]           M_AXI_BRESP;

wire                  M_AXI_BREADY;

// Instantiate the Device Under Test (DUT)

// Note: Ensure your dmac_read, dmac_buffer, and dmac_write are compiled alongside this.

axi_dma_controller #(

.ADDR_WD(ADDR_WD),

.DATA_WD(DATA_WD),

.CHANNEL_COUNT(CHANNEL_COUNT),

.MAX_BURST_LEN(MAX_BURST_LEN)

) dut (.*);

// Clock Generation (100MHz)

initial begin

clk = 0;

forever #5 clk = ~clk;

end

// VCD Dumping for Waveform Viewer

initial begin

$dumpfile("axi_dma_waveform.vcd");

$dumpvars(0, tb_axi_dma_controller);

end

// Main Test Sequence

initial begin

// Initialize Inputs

rst = 1;

cmd_valid = 0;

cmd_src_addr = 0;

cmd_dst_addr = 0;

cmd_burst = 0;

cmd_len = 0;

cmd_size = 0;

M_AXI_ARREADY = 0;

M_AXI_RVALID  = 0;

M_AXI_RDATA   = 0;

M_AXI_RRESP   = 0;

M_AXI_RLAST   = 0;

M_AXI_AWREADY = 0;

M_AXI_WREADY  = 0;

M_AXI_BVALID  = 0;

M_AXI_BRESP   = 0;

// Apply Reset

#20 rst = 0;

#10;

// Issue DMA Command (e.g., Transfer 2 beats of 32-bit data)

@(posedge clk);

cmd_valid    <= 1;

cmd_src_addr <= 32'h1000_0000;

cmd_dst_addr <= 32'h2000_0000;

cmd_burst    <= 2'b01; // INCR

cmd_len      <= 1;     // 2 beats (Length = beats - 1)

cmd_size     <= 3'b010; // 4 bytes per beat

// Wait for DUT to accept command

wait(cmd_ready == 1'b1);

@(posedge clk);

cmd_valid <= 0;

// Wait enough time for the automated AXI responders (below) to finish

#500;

$display("Simulation Complete.");

$finish;

end

// ==========================================

// Automated AXI Slave Responders

// ==========================================

// 1. Mock AXI Read Address Channel

always @(posedge clk) begin

if (M_AXI_ARVALID && !M_AXI_ARREADY) begin

M_AXI_ARREADY <= 1; // Accept address

end else begin

M_AXI_ARREADY <= 0;

end

end

// 2. Mock AXI Read Data Channel

always @(posedge clk) begin

if (M_AXI_ARVALID && M_AXI_ARREADY) begin

// Wait a cycle then send data

@(posedge clk);

M_AXI_RVALID <= 1;

M_AXI_RDATA  <= 32'hAAAA_1111; // Beat 1

M_AXI_RLAST  <= 0;

wait(M_AXI_RREADY);

@(posedge clk);

M_AXI_RVALID <= 1;

M_AXI_RDATA  <= 32'hBBBB_2222; // Beat 2 (Last)

M_AXI_RLAST  <= 1;

wait(M_AXI_RREADY);

@(posedge clk);

M_AXI_RVALID <= 0;

M_AXI_RLAST  <= 0;

end

end

// 3. Mock AXI Write Address Channel

always @(posedge clk) begin

if (M_AXI_AWVALID && !M_AXI_AWREADY) begin

M_AXI_AWREADY <= 1; // Accept write address

end else begin

M_AXI_AWREADY <= 0;

end

end

// 4. Mock AXI Write Data Channel

always @(posedge clk) begin

if (M_AXI_WVALID && !M_AXI_WREADY) begin

M_AXI_WREADY <= 1; // Accept write data

end else begin

M_AXI_WREADY <= 0;

end

end

// 5. Mock AXI Write Response Channel

always @(posedge clk) begin

if (M_AXI_WVALID && M_AXI_WREADY && M_AXI_WLAST) begin

// Send response after last beat

@(posedge clk);

M_AXI_BVALID <= 1;

M_AXI_BRESP  <= 2'b00; // OKAY

wait(M_AXI_BREADY);

@(posedge clk);

M_AXI_BVALID <= 0;

end

end

endmodule

WaveForm